# Orbit Speak Quick Reference Command list

Power on = Press and hold the power key for two seconds to turn on the unit.

Up Arrow = Up Arrow / Space + Dot 1

Down Arrow = Down Arrow / Space + Dot 4

Select = Select Key / Dot 8

Go to Top of the list = Long press up arrow / Space + Dot 1 2 3

Go to Bottom of the list = Long press down arrow / Space + Dot 4 5 6

Open sub menu = Select Key / Dot 8 / Right Arrow

Close sub menu = Left Arrow / Space + Dot 1 5 / Space + Dot 1 3 5 6

Open File Manager = f

Open Editor = e

Open Media Player = m

Open Settings = s

Open Utilities sub menu = u

Open Help sub menu = h

## Utilities Menu items

Check battery status = b / space + Dot 1 6

Check current time = t / space + t (Dot 2 3 4 5)

Check current date = d / space + t twice (space + Dot 2 3 4 5)

Calculator = c

Alarm & Clock = a

Contacts = o

Format SD Card = f

## Help Menu Items

User guide = u

Audio tutorial = t

General Commands

Sleep/wakeup = Short press power key (Single press to Power key)

Power ON/OFF = Long press power key (Press Power Key min 2 seconds)

Mass storage enter/exit =  Space + Dot 7 5

Processor volume down = F1

Processor volume up = F2

Increase speech volume = Dot 7 + F4

Decrease speech volume = Dot 7 + F1

Battery Status = Space + Dot 1 6

Repeat last announcement = Space + Dot 1 4

Android reset = F4 + Up key arrow simultaneous for 15 sec

## Global Hot Keys

Home Menu = Space + Select + Dot 1 /  Space + Dot 1 2 3 4 5 6

File Manager = Space + Select + Dot 2  or F1+ F (Dot 1 2 4)

Settings = Space + Select + Dot 1 6

Editor = Space + Select + Dot 1 5

Media Player = Space + Select + Dot 1 4

Calculator = Space + Select + Dot 4 or F1+t

Alarm & Clock = Space + Select + Dot 6 or F1 + C

## Global Commands

Increase Speech Rate = Space + Select + Dot 4 5 / F4 + Space

Decrease Speech Rate = Space + Select + Dot 1 2 / F1 + Space

Increase Pitch Rate = Space + Select + Dot 4 6 / F4 + Dot 8

Decrease Pitch Rate = Space + Select + Dot 1 3 /  F1 + Dot 8

Play Audio Tutorials = Long Press F4

Current Date = Press twice  Space + Dot 2 3 4 5

Current Time = Space + Dot 2 3 4 5

## File Manager Commands

Note: Access Keys mentioned in the table below are when you are in a menu

Select Previous File = Up Arrow / Space + Dot 1

Select Next file = Down Arrow / Space + Dot 4

Open File/Folder = Select Key or Dot 8 or Access Key O (Dot 1 3 5)

File/Folder Properties = Dot 8 2 4 or Access key I (Dot 2 4)

Navigate to previous folder = Dot 7

Exit File Manager = Space 1 3 5 6 or Access Key X (Dot 1 3 4 6)

Open Context Menu = Space + m (Dot 1 3 4)

Open Sub-menus = Select, Dot 8, f for file, e for edit, v for view

Close Context Menu = Space + z (Dots 1-3-5-6)

Mark/Unmark File/Folder = Space or Access Key M (Dot 1 3 4)

Copy = Dot 8 1 4 or Access Key Y (Dot 1 3 4 5 6)

Cut = Dot 8 1 3 4 6 or Access Key C (Dot 1 4)

Paste = Dot 8 1 2 3 6 or Access Key E (Dot 1 5)

Delete = Space + D (Dot 1 4 5) or Access Key D (Dot 1 4 5)

Confirm Delete = Select or Dot 8 / Access Key Y

Cancel =  = Space 3

Top of the current list = Long Press UP Arrow / Space + Dot 1 2 3

Bottom of the current list = Long Press Down Arrow / Space + Dot 4 5 6

Create a new Folder = Dot 8 1 2 4 or Access Key N (Dot 1 3 4 5)

Rename a folder/file = Space + Dot 1 2 3 5 or Access Key R (Dot 1 2 3 5)

Open Search Box = Space + Dot 1 2 4 or Access Key F (Dot 1 2 4)

Begin Search = Dot 8

Navigate in the dialog = Up/Down arrow or Space + Dot 1/Space + Dot 4 or tab, Space + Dot 4 5 or Shift Tab, Space + Dot 1 2

Remove inserted character = Dot 7

Select an option in the dialog = Select Key

Select all = Dot 8 1 or Access Key A

Sort By = Access Key S

Order By = Access Key O

Show file types = Access Key T

Close all dialogs = Space + E (Dot 1 5)

## File Picker Dialog Commands

Select Prev/Next File = Up Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Open File/Folder = Select Key or Dot 8

File/Folder Properties = Dot 8 2 4

Navigate through available network list = Up/Down arrows or Space + Dot 1/ Dot 4

Navigate through components = Space + Dot 4 5 or Space + Dot 1 2

Perform action =  = Select or Dot 8

Navigate to previous folder = Dot 7

Close dialog = Space + 1 3 5 6 or Space + 1 5 or Dot 7

## Editor Application Commands

Next Word = Space + Dot 5 or space + Right Arrow

Previous Word = Space + Dot 2 or Space + Left Arrow

Next Character = Space + Dot 6 or Right Arrow

Previous Character = Space + Dot 3 or Left Arrow

Previous Line = Space + Dot 1 or Up Arrow

Next Line = Space + Dot 4 or Down Arrow

Move To BEGINNING Of Line = Space + k (Dot 1 3)

Move To end of Line = Space + Dot 4 6

Move To Top of Document = Space + l (Dot 1 2 3)

Move To Bottom of Document = Space + Dot 4 5 6

Start/Stop Selection Mode = Dot 1 2 8

Delete Character before cursor = Dot 7

Delete Character at cursor = Space + d (Dot 1 4 5)

Add A New Line = Dot 8 or Select

Copy Selection = Dot 1 4 8

Cut Selection = Dot 1 3 4 6 8

Paste Selection = Dot 1 2 3 6 8

Select All = Dot 1 8

Find = Space + Dot 1 2 4

Find Next result = Space + Dot 1 2 4 8

Find Previous Result = Space + Dot 1 2 4 7

Go To Line = Space + Dot 1 2 4 5

Read Current Line (Pressing twice quickly spells it) = Space + Dot 1 4

Read Current Word (Pressing twice quickly spells it) = Space + Dot 2 5

Read Current Character = Space + Dots 3 6

Say All From Cursor To End = Dot 1 2 4 5 8

Read From Start To Cursor = Space + Dot 1 2 4 5 7

Read Selected Text = Space + Dot 1 2 7

Open Context Menu = Space + m (Dot 1 3 4)

Create New Document = Dot 1 3 4 5 8

Open a Document = Dot 1 3 5 8

Save Document = Dot 2 3 4 8

Save As = Space + s (Dot 2 3 4)

Exit Editor = Space + z (Dot 1 3 5 6)

## Media Player Application Commands

Open File Manager = Dot 8 1 3 5

Play/Pause = Select or Space

Apply next selected movement unit = Right arrow

Apply previous selected movement unit = Left arrow

Previous movement unit = Up arrow

Next movement unit = Down arrow

Shuffle Mode toggle = Dot 1 2 5 8

Media Info = Dot 8 2 3 4 5

Open Context Menu = Space + m (Dot 1 3 4)

Navigate Context Menu Items = Up Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Enter Inside Sub Menu = Dot 8 or Right arrow

Exit From Sub Menu = Dot 7 or Left arrow

Navigate Sub Menu Values = Up Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Apply Sub Menu Values = Select

Close Context Menu or Settings Dialog = Space + e (Dot 1 5) or Space + z (Dot 1 3 5 6)

Close Media Player app = Space + Dot 1 3 5 6

Media Time Info = Dot 2 3 4 5 8

Previous / Next Track = Dot 1 / Dot 4

Increase / Decrease Playback speed = Dot 6 / Dot 3

Toggle Repeat mode = Dot 1 2 3 5 8

Increase / Decrease volume = F2 or Dot 5 / F1 or Dot 2

Open settings dialog = Dot 2 3 4 8

## Settings:

Open sub-settings = Right arrow or space+ Dot 6 or Select key Dot 8

Access keys:

[o] for general options

[s] for speech options

[w] for Wi-Fi settings

[b] for Bluetooth settings

[v] for advance options

[a] for about

Navigate to different combo box = Up/Down arrows or Space + 1 / Space + 4 or Space + Dot 1 2 / Space + Dot 4 5

Navigate between the values in the combo box = Left/Right arrow

Apply all combo box values = Select or Dot 8

Close dialog = Space + Dot 1 5 / Space + Dot 1 3 5 6

First letter navigation Works in about dialog and speech options combo box

## Wi-Fi Setting

Navigate through available network list = Up/Down arrows or Space + Dot 1/ Dot 4

Navigate through components = Space + Dot 4 5 or Space + Dot 1 2

Perform action = Select or Dot 8

Enable/Disable Wi-Fi = Dot 8 + w

Refresh available network list = Dot 8 + r

Navigate to top of available network list = Space + Dot 1 2 3

Navigate to bottom of available network list = Space + Dot 4 5 6

Enable Wi-Fi dialog: Navigate through components = Space + Dot 4 5/Dot 1 2 or Up/Down arrows or Space + Dot 1/ Dot 4

Enable Wi-Fi dialog: Perform action = Select or Dot 8 or Access key y or n

Enable Wi-Fi dialog: Close = Space + 1 3 5 6 or Space + 1 5 or Access key n

Context Menu: Open = Select or Dot 8 on available network

Context Menu: Close = Space + 1 3 5 6 or Space + 1 5 or Dot 7

Context Menu: Access key = Connect c or Forget f

Context Menu: Navigate through menus = Up/Down arrows or Space + Dot 1/ Dot 4

Close Wi-Fi Settings dialog = Space + 1 3 5 6 or Space + 1 5 or Dot 7

Network Security Dialog: Navigation = Space + Dot 4 5/Dot 1 2 or Up/Down arrows or Space + Dot 1/ Dot 4

Network Security Dialog: Perform action = Select or Dot 8

## Bluetooth Setting

Navigate through available device list = Up/Down arrows or Space + Dot 1/ Dot 4

Navigate through components = Space + Dot 4 5 or Space + Dot 1 2

Perform action = Select or Dot 8

Enable/Disable Bluetooth = Dot 8 + b

Refresh available device list = Dot 8 + r

Navigate to top of available device list = Space + Dot 1 2 3

Navigate to bottom of available device list = Space + Dot 4 5 6

Enable Bluetooth dialog:

Navigate through components = Space + Dot 4 5 / Dot 1 2 or Up/Down arrows or Space + Dot 1/ Dot 4

Perform action = Select or Dot 8 or Access key y or n

Close = Space + 1 3 5 6 or Space + 1 5 or Access key n

Context Menu = space + m (Dots 1 3 4)

Open  = Select or Dot 8 on available device

Close = Space + 1 3 5 6 or Space + 1 5 or Dot 7

Access key = Connect c, Forget f

Navigate through menus = Up/Down arrows or Space + Dot 1 / Dot 4

Close Bluetooth Settings dialog = Space + 1 3 5 6 or Space + 1 5 or Dot 7

## Calculator

Remove Character = Dot 7

Evaluate expression (Get Result) = Dot 8

Clear whole display = Space + d (Dot 1 4 5)

Open Context Menu = Space + m (Dot 1 3 4)

Access Key of Plus in context menu = p (Dot 1 2 3 4)

Access Key of Minus in context menu = m (Dot 1 3 4)

Access Key of Multiply in context menu = l (Dot 1 2 3)

Access Key of Divide in context menu = d (Dot1 4 5)

Access Key of Clear Display in context menu = c (Dot 1 4)

Access Key of Exit App in context menu = z (Dot 1 3 5 6)

Navigate up/down in context menu = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate to top of list = Long Press Up or Space + Dot 1 2 3

Navigate to bottom of list in context menu = Long Press Down or Space + Dot 4 5 6

Exit from context menu = Space + z (Dot 1 3 5 6) or Space + e (Dot 1 5)

Exit from app = Space + z (Dot 1 3 5 6)

## Alarm Clock

Navigate Up/Down in alarm list or edit alarm dialog options = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate Up/Down in the new alarm dialog options or delete alarm dialog option = UP Arrow or Space + Dot 1 or tab (space + Dot 4 5) / Down Arrow or Space + Dot 4 or shift-tab (space + Dot 1 2)

Open Context Menu = Space + m (Dot 1 3 4)

Access Key of New Alarm in context menu = n (Dot 1 3 4 5)

Access Key of Edit Alarm in context menu = e (Dot 1 5)

Access Key of Toggle Alarm in context menu = o (Dot 1 3 5)

Access Key of Delete Alarm in context menu = d (Dot 1 4 5)

Access Key of Exit App in context menu = z (Dot 1 3 5 6)

Navigate up/down in context menu = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate to top of list in context menu of alarm dialogs = Long Press Up or Space + Dot 1 2 3

Navigate to bottom of list in context menu of alarm dialogs = Long Press Down or Space + Dot 4 5 6

Exit from any dialog = Space + z (Dot 1 3 5 6) or Space + e (Dot 1 5)

Exit from app = Space + z (Dot 1 3 5 6)

Open New alarm = Enter + n (Dot 8 1 3 4 5)

Open Edit alarm = Enter + e (Dot 8 1 5)

Open Delete alarm = Space + d (Dot 1 4 5)

Toggle Alarm = Space

Navigate to left or right in the combo box in the new alarm dialog & edit alarm dialogs = Left Arrow or Space + Dot 3 / Right Arrow or Space + Dot 6.

Access key in the delete dialog to confirm delete = y (Dot 1 3 4 5 6)

Access key in the delete dialog to cancel delete = n (Dot 1 3 4 5)

Navigate Up/Down in the Alarm triggering dialog = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate Top of list/Bottom of list in the Alarm triggering dialog = Long Press Up or Space + Dot 1 2 3 / Long Press Down or Space + Dot 4 5 6

Select option in alarm triggering dialog or delete dialog = Dot 8 or Select Key

Exit App = Space + z (Dot 1 3 5 6)

Dismiss all alarms = Space + z (Dot 1 3 5 6)

Dismiss the triggering alarm = Space + e (Dot 1 5)

## Contacts

Navigate Up/Down in contact list = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate Up/Down in dialog = UP Arrow or Space + Dot 1 or tab (space + Dot 4 5) / Down Arrow or Space + Dot 4 or shift-tab (space + Dot 1 2)

Open Context Menu = Space + m (Dot 1 3 4)

Access Key of New Contact in context menu = n (Dot 1 3 4 5)

Access Key of Edit Contact in context menu = e (Dot 1 5)

Access Key of Delete Contact in context menu = d (Dot 1 4 5)

Access Key of Exit App in context menu = z (Dot 1 3 5 6)

Navigate up/down in context menu = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate top of list/bottom of contact list = Long Press Up or Space + Dot 1 2 3 / Long Press Down or Space + Dot 4 5 6

Exit from any dialog = Space + z (Dot 1 3 5 6) or Space + e (Dot 1 5)

Exit from app = Space + z (Dot 1 3 5 6)

New contact = Enter + n (Dot 8 1 3 4 5)

Edit contact = Enter + e (Dot 8 1 5)

Delete contact = Space + d (Dot 1 4 5)

View Contact = Select or Dot 8

Save changes in edit and new contact dialog = Select or Dot 8

Confirm dialog: access key Yes = y (Dot 1 3 4 5 6)

Confirm dialog: access key No = n (Dot 1 3 4 5)

Navigate Top of list/Bottom of list in the Alarm triggering dialog = Long Press Up or Space + Dot 1 2 3 / Long Press Down or Space + Dot 4 5 6

Exit App = Space + z (Dot 1 3 5 6) or Space + e (Dot 1 5)

## Calendar

Navigate Up/Down/Left/Right in calendar view = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4 / Left Arrow or Space + 3 / Right Arrow or Space + 6

Open Context Menu = Space + m (Dot 1 3 4)

Access Key of New Event in context menu = n (Dot 1 3 4 5)

Access Key of Edit Event in context menu = e (Dot 1 5)

Access Key of View Event in context menu = v (Dot 1 2 3 6)

Access Key of Delete Event in context menu = d (Dot 1 4 5)

Access Key of Go to Date in context menu = g (Dot 1 2 4 5)

Access Key of Exit App in context menu = z (Dot 1 3 5 6)

Navigate up/down in the new event = edit event = context menu = go to date = delete event = event triggered dialog & event details page = UP Arrow or Space + Dot 1 or Space + Dot 1 2 / Down Arrow or Space + Dot 4 or Space + Dot 4 5

Navigate top of list/bottom of list in context menu, new event, edit event, event list,  go to date dialog, delete event dialog, event triggered dialog, event details page = Long Press Up or Space + Dot 1 2 3 / Long Press Down or Space + Dot 4 5 6

Exit from any dialog = Space + z (Dot 1 3 5 6) or Space + e (Dot 1 5)

Exit from app = Space + z (Dot 1 3 5 6)

Open New event dialog = Enter + n (Dot 8 1 3 4 5)

Open Edit event dialog = Enter + e (Dot 8 1 5)

Open Delete event dialog = Space + d (Dot 1 4 5)

Open go to date dialog = Space + g (Dot 1 2 4 5)

Navigate items in the combo box in the new event dialog,  edit event dialog or go to date  = Left Arrow or Space + Dot 3 / Right Arrow or Space + Dot 6.

Access key in the delete dialog to confirm delete = y (Dot 1 3 4 5 6)

Access key in the delete dialog to cancel delete = n (Dot 1 3 4 5)

Navigate week wise in the calendar view = UP Arrow or Space + Dot 1 / Down Arrow or Space + Dot 4

Navigate to previous/next date in the calendar view = Left Arrow or Space + Dot 3 / Right Arrow or Space + Dot 6.

Navigate to previous/next month in the calendar view = Dot 8 + Left Arrow / Dot 8 + Right Arrow

Navigate to previous/next year in the calendar view = Space + Dot 8 + p (Dot 1 2 3 4) / Space + Dot 8 + n (Dot 1 3 4 5)

Select option in all dialogs = Dot 8 or Select Key

Dismiss the triggering alarm = Space + e (Dot 1 5) or Space + z (Dot 1 3 5 6)

Open Event Details page = Dot 8 or Select Key

## Battery Logging

Navigate Up/Down in the config dialog = UP Arrow or Space + Dot 1 or tab (space + Dot 4 5) / Down Arrow or Space + Dot 4 or shift-tab (space + Dot 1 2)

Navigate Top/Bottom in the config dialog = Long Press Up or Space + Dot 1 2 3 / Long Press Down or Space + Dot 4 5 6

Open config dialog = F1 + F2 + Dot 7 + Dot 8 + F3 + F4

Start/Stop logging = F1 + F2 + Space + F3 + F4

Remove inserted digit = Dot 7

Select option in dialog = Dot 8 or Select

Dismiss dialog = Space + e (Dot 1 5) or Space + z (Dot 1 3 5 6)

## Internet Radio

Navigate between category list and item list = Space + Dot 1 2 / Space + Dot 4 5

Open search station by name dialog = Space + Dot 1 2 4

Back to country list or category list = Dot 7

Close Internet Radio = Space + Dot 1 3 5 6 or Space + Dot 1 5

Top of current list = Space + Dot 1 2 3

Bottom of current list = Space + Dot 4 5 6

Navigate in current list = Space 1/Space 4 or Up/Down

Select item in current list = Select or Dot 8

Add station from URL = Space + Dot 1 3 4 5

Delete my station or favorite = Space + Dot 1 4 5

Open context menu = Space + Dot 1 3 4

Search for station = Access key: s

Add station from URL = Access key: u

Play station = Access key: p

Add or remove favorite = Access key: f

Rename = Access key: r

Delete = Access key: d

Station information = Access key: i

Exit = Access key: z

Close context menu = Dot 7 or Space + Dot 1 3 5 6 or Space + Dot 1 5

Player Dialog: Play/Stop = Space or Select

Close dialogs = Space + Dot 1 3 5 6 or Space + Dot 1 5

Increase/Decrease volume = F2 or Dot 5 / F1 or Dot 2

Start/Stop recording = F3 *Alert, this will record from the built-in microphone. For better recording, use the context menu space + m then arrow down to record/stop recording, and press enter, dot 8.

Navigate to the items of the combo box in the settings dialog, Navigate to the main items in the settings dialog = Up/Down Arrow key.

## Reader

Navigate between category list and item list = Space + Dot 1 2 / Space + Dot 4 5

Close dialogs = Space + Dot 1 3 5 6 or Space + Dot 1 5

Back to category list = Dot 7

Close Reader = Space + Dot 1 3 5 6 or Space + Dot 1 5

Top of current list = Space + Dot 1 2 3

Bottom of current list = Space + Dot 4 5 6

Navigate in current list = Space 1/Space 4 or Up/Down

Open book or Select item in current list = Select or Dot 8

Add a local book = Space + Dot 1 3 4 5

Add an online service = Dot 8 1

Delete a local book, a downloaded book or an online service = Space + Dot 1 4 5

Open online book details = Dot 8 2 4

Open search = Space + Dot 1 2 4

Open context menu = Space + Dot 1 3 4

Close context menu = Dot 7 or Space + Dot 1 3 5 6 or Space + Dot 1 5

## Main Page Context Menu:

Clear Recent = Access key: C

Add a book = Access key: a

Scan files = Access key: s

Delete book = Access key: d

Open book = Access key: o

Exit = Access key: x

Add online service = Access key: l

Delete and logout = Access key: d

Book information = Access key: i

Download = Access key: d

Search = Access key: s

Speech Options = Access key: p

Settings = Access key: t

## Book Reader dialog:

Book Reader context menu

File = Access key: f

Open book = Access key: o or Dot 8 1 3 5

Close book = Access key: c or Space + Dot 1 5

Close book and clear reading position = Access key: p or Dot 8 1 4

Speech options = Access key: s

Exit = Access key: x

Goto = Access key: g

Find = Access key: f or Space + Dot 1 2 4

Find Next = Access key: n or Space + Dot 8 1 2 4

Find Previous = Access key: p or Space + Dot 7 1 2 4

Go to location = Access key: g or Space + Dot 1 2 4 5

Reading = Access key: r

Toggle continuous reading = Access key: c or Select or Dot 8 or Space

Read current paragraph = Access key: r or Space + Dot 1 2 3 4

Read current sentence = Access key: s or Space + Dot 2 3 5 6

Bookmark = Access key: k

Set bookmark = Access key: s or Dot 8 1 3 4

b) Jump to bookmark = Access key: j or Dot 8 2 4 5

c) Delete bookmark = Access key: d or Space + 1 4 5

Bookmark list = Access key: l or Dot 8 1 3

Play/Stop = Space or Select or Dot 8

Increase/Decrease Volume = Dot 2 Dot 5

Increase/Decrease Playback speed = Dot 3 Dot 6

Navigate to start or end of file. Space + Dot 1 2 3 or Space + Dot 4 5 6